Novell's IPX/SPX

The Novell protocol suite is named after its two primary protocols: Internet Packet Exchange (IPX) and Sequenced Packet Exchange (SPX). This proprietary protocol stack is based on Xerox's Network Systems (XNS) protocol, used with first generation Ethernets. IPX/SPX became prominent during the early 1980s as an integral part of Novell, Inc.'s NetWare. NetWare became the de facto standard network operating system (NOS) of first generation LANs. Novell complemented its NOS with a business-oriented application suite and client-side connection utilities.

IPX is much like IP. It is a connectionless datagram protocol that does not require, nor provide, an acknowledgment for each packet transmitted. IPX also relies upon SPX in the same way that IP relies upon TCP for sequencing and other Layer 4 connection-oriented services. The relationship between the OSI Reference Model and the IPX/SPX protocol suite is illustrated in Figure 4.4.

FIGURE 4.4. Comparison of the OSI Reference Model and IPX/SPX.

Novell's IPX and SPX protocols provide functionality equivalent to the OSI Layers 3 and 4, respectively. The full suite of IPX/SPX protocols provides the functionality of the other OSI layers in four layers.

Dissecting IPX/SPX

The IPX/SPX protocol stack includes four functional layers: application, internet, data link, and medium access. These four layers are presented from the highest to the lowest, and loosely correlate to the seven layers of the OSI Reference Model without compromising functionality.

Application Layer

Novell's application layer encompasses the application, presentation, and session layers of the OSI model, although some of its application protocols extend as far down the stack as the OSI network layer. The primary application layer protocol in this stack is the NetWare Core Protocol (NCP). NCP can interface directly with both SPX and IPX. NCP is used for printing, file sharing, e-mail, and directory access.

Other application layer protocols include Routing Information Protocol (RIP), a proprietary Service Advertisement Protocol (SAP), and NetWare Link Services Protocol (NLSP), among others.

RIP is the default routing protocol for NetWare. It is a distance-vector routing protocol that uses only two metrics: ticks and hops. A tick is a measure of time, and a hop, as explained earlier in this chapter, is the running tally of routers that have handled the routed packet. These two metrics are the basis for IPX routing path decisions. Ticks are the primary metric for determining paths. Hops are used only as "tie breakers" in the event of two or more paths having the same tick value.

RIP is a very simple and mature routing protocol. In addition to its limited number of distance-vector metrics, it suffers from a high level of network overhead incurred because RIP routing table updates are broadcast every 60 seconds. This can have adverse effects on large or very busy networks.

SAP is a unique and proprietary protocol that Novell has successfully used to enhance the client/server relationship. Servers use SAP to automatically broadcast their available services throughout the network immediately after becoming active on the network. They periodically broadcast SAPs to keep clients and other servers informed of their status and services.

SAP broadcasts include the server's name and type, operational status, and network, node, and socket numbers. Routers can store information from an SAP broadcast and propagate it to other network segments. Clients can also initiate SAP requests when they need a specific service. Their request is broadcast throughout the network segment. Hosts can then respond and provide the client with enough SAP information to determine whether or not the service is available within a reasonable distance.

Unfortunately, SAP is an extremely mature protocol that is becoming increasingly ill-suited to functioning in contemporary networks. As with RIP, service advertisements occur every 60 seconds. On today's large, flat, switched LANs, this degree of broadcasting can be problematic.

The newest application layer protocol is NetWare Link Services Protocol (NLSP). NLSP is a link-state routing protocol that Novell intends to use as a replacement for the aging RIP and SAP protocols. NLSP updates routes only when changes have been made.

Internet Layer Protocols

The Internet layer of IPX/SPX maps loosely to both the network and transport layers of the OSI Reference Model. IPX is predominantly a Layer 3, or network layer, protocol although it is capable of directly interfacing with the application layer. SPX is distinctly a Layer 4, or transport layer, protocol and cannot directly interface with the data link layer's ODI. It must pass data through IPX and let IPX interface with the ODI. IPX and SPX function as sublayer protocols within a common Internet layer.

SPX is connection-oriented and can be used to transmit data between a client and server, two servers, or even two clients. As with TCP, SPX provides reliability to IPX transmissions by managing the connection, providing flow control, error checking, and packet sequencing.

The SPX header has the following size and structure:

- Connection Control: The first octet (eight bits) of the SPX header provides four 2-bit flags that control the bi-directional flow of data across an SPX connection.

- Datastream Type: The next eight bits of the header define the type of datastream.

- Source Connection Identification: The 16-bit Source Connection Identification field is used to identify the process responsible for initiating the connection.

- Destination Connection Identification: The 16-bit Destination Connection Identification field is used to identify the process that accepted the inbound SPX connection request.

- Sequence Number: The 16-bit Sequence Number field provides the destination host's SPX protocol with a count of packets transmitted. This sequential numbering can be used to reorder the received packets, should they arrive out of sequence.

- Acknowledgment Number: The Acknowledgment Number field is a 16-bit field that indicates the next expected packet.

- Allocation Number: The Allocation Number field is 16 bits in length and is used to track the number of packets sent, but not yet acknowledged, by the intended recipient.

- Data: The last field in the SPX header contains the data. Up to 534 octets of data may be transmitted, per SPX packet.

Novell's other Internet layer protocol is IPX. IPX provides a connectionless, best-effort, datagram delivery service. It prepares SPX, or other protocols', packets for delivery across multiple networks by prepending an IPX header to them. This new structure is called an IPX datagram. This datagram's header contains all the information necessary to route the packets to their destination, regardless of where that might be.

The IPX header is 11 octets long and has the following structure:

- Checksum Field: The IPX header begins with a 16-bit legacy field that exists solely to provide backward compatibility with its ancestral XNS protocol. XNS used this field for error checking. IPX defaults this field to "FFFFH" and trusts higher-level protocols to detect (and correct) any transmission errors.

- Packet Length Field: This 16-bit field defines the length of the IPX datagram, including the header and data. Packet Length is checked to verify packet integrity.

- Transport Control: This 8-bit field is used by routers during the forwarding of the datagram. IPX sets it to 0 prior to transmission. Each router that receives and forwards the datagram increments this field by 1.

- Packet Type: This 8-bit field identifies the type of packet that is embedded in the IPX datagram. This field allows the destination host to pass the contents to the next appropriate protocol layer. Types can include RIP, NCP, SPX, error, and so on.

- Destination Network Number: This 32-bit field identifies the network number of the destination node.

- Destination Node: This 48-bit field contains the node number of the destination machine.

- Destination Socket Number: As IPX allows for multiple simultaneous connections to one system, it is essential to identify the socket number of the process or program receiving the packets. This 16-bit field provides this information.

- Source Network Number: This 32-bit field identifies the network number of the source node.

- Source Node Address: This 48-bit field contains the node number of the source machine.

- Source Socket Number: This 16-bit field identifies the socket number of the process or program sending the packets.

Typical IPX/SPX Operation

SPX creates and maintains a connection-oriented bit stream between two networked devices. The protocol accepts large blocks of data from higher-level protocols and breaks them into more manageable pieces that are up to 534 octets long. The header just described is prepended to the data to create SPX packets. These packets are passed to the Internet sublayer protocol IPX.

The segment's header fields are populated appropriately, and the segment is passed to IPX. IPX adds the network addressing, length, checksum, and other header information before passing the packet on to the data link layer. Figure 4.5 depicts an IPX/SPX packet in this state, just prior to being passed to the data link layer.

FIGURE 4.5. Structure of an 802.3 Ethernet frame encap- sulating an IPX/SPX payload.

This illustration shows the positioning of the IPX and SPX headers within an 802.3 Ethernet frame. This is the structure used to pass data between the two sublayers of Novell's Internet layer.

The destination machine performs the reverse of the operation just described. It receives the packets and passes them to its SPX protocol for reassembly. If necessary, the packets are reordered into data segments that are passed up to the appropriate application.

Data Link and Medium Access Layers

NetWare's equivalents of the OSI physical and data link layers are the medium access and data link layers. The data link layer is directly compatible with the Open Data-Link Interface (ODI) standard. Similarly, the medium access layer is directly compatible with all common, standardized media access protocols.

This low-level adherence to open industry standards enables NetWare and the IPX/SPX protocol stack to be implemented almost universally.

IPX Addressing

IPX addresses are 10 octets (80 bits) long. This is significantly larger than IPv4's 32-bit address, but less than IPv6's 128-bit address. Each address is comprised of two components: a network number up to 32 bits in length and a 48-bit node number. These numbers are expressed in dotted hexadecimal notation. For example, 1a2b.0000.3c4d.5e6d could be a valid IPX address where the 1a2b represented the network number and 0000.3c4d.5e6d was the node number.

IPX addresses, too, can be "invented" by a network administrator. However, this runs the risk of address conflicts when internetworking. Invention of network numbers also places the burden of maintaining and managing all such fabricated numbers on the network administrator. The better approach is to obtain registered IPX network numbers from Novell.

The universally assigned address (MAC address) on the network interface card (NIC) is usually used as the IPX host number. Given that these addresses are, at least in theory, unique, this provides a convenient, unique host numbering.

As with IP, IPX is capable of supporting multiple simultaneous sessions. This creates the need for identifying the specific process or program that is communicating on any given session. This identification is achieved through the use of a 16-bit socket number in the IPX header. This socket number is analogous to TCP/IP's port number.

IPX/SPX Conclusion

Novell, Inc. has watched the market share of its proprietary IPX/SPX protocol stack plummet under competitive pressure. As "open" protocol stacks such as OSI, IP, and others became available, IPX/SPX suffered. Commercially available office automation software bundles also cut into Novell's sales. Having a proprietary, tightly coupled series of products became viewed as a liability in a marketplace that valued openness and interoperability.

Novell has demonstrated its commitment to regaining its lost prominence by making IPv6 its default protocol for future versions of NetWare. To successfully implement this change of strategy, Novell must ensure backward compatibility of IPv6 and IPX/SPX. To achieve this goal, Novell has worked closely with the Internet Engineering Task Force (IETF) during the design of IPv6. This has resulted in numerous IPX services becoming integral to IPv6.

Having set the stage for its future, Novell now must ensure a seamless migration of its current protocol stack and application suite to this new environment. More importantly, it needs to provide value-adding products and services using an open network platform. Novell's vision for the future is to provide Network Directory Services (NDS) and related products for two customer bases: the Internet community and corporate intranet communities.

NDS provides a single, global, logical view of all network services and resources. This allows users to access network services and resources with a single login, regardless of the user's location or the location of the resources.

(www.developer.com)